Sultanate of Oman

Ministry of Agriculture, Fisheries & Water Resources

Consultancy Services for Design Review and Supervision of 
Wadi Majlas Dam in Wilayat Qurayat, Muscat Governorate

Wadi Majlas Flood Protection Dam

Hydrology Report

Rev 00

July 2026

Table of contents

Executive Summary	5

Project Context	5

Scope of Work	5

Hydrological Approach	5

Hydraulic Design Methodology	5

Key Findings	6

Conclusions and Recommendations	6

Hydraulic Design of Channels– Block 6	7

1.1.	Hydrology	10

1.2.	Channel alignments	14

1.3.	Channel condition and conflicts	17

1.4.	Rainfall runoff simulation	19

1.4.1.	Dams’ information	23

1.4.2.	Dams’ elevation-volume-area curves	24

1.5.	Hydraulic	31

1.5.1.	Channel Design Criteria	31

1.5.2.	Main issues in channel design	34

1.5.3.	Channel depth	35

1.5.4.	Channels design	36

1.6.	Outfalls	49

1.7.	Culvert/Bridge	51

1.8.	Dykes	64

1.9.	Conclusion	66

List of Tables

Table 1- Channel Design Process – Block 6	7

Table 2- Design Rainfalls	10

Table 3- Area reduction factor (ARF)	12

Table 4- Dams’ information	24

Table 5- Dams’ elevation-volume-area curves	25

Table 6- Dams’ Peak flow for different return periods	27

Table 7- Channel Design Criteria	31

Table 8- Mean tidal levels in North and South Al Batinah (m) from Oman Maritime Book 2024	32

Table 9- Challenges in design of channels	34

Table 10- Channels and dykes length in Block 06	36

Table 11- Channel sizes inventory	38

Table 12- Channel characteristics	42

Table 13- Number of cross drainage structures by type	51

Table 14- cross drainage structures detail	51

Table 15- Existing cross drainage structures’ photos	55

Table 16- Dykes characteristics	64

Table of Figures

Figure 1- Channel Design Process – Block 6	9

Figure 2-PMP grid	11

Figure 3- Rainfall Distribution Patterns	12

Figure 4 – Aea Reduction Factor Curve	13

Figure 5- Curve Number grid	14

Figure 6 – Channel Category Classification – Block 6	15

Figure 7- Channel alignments to control flood	16

Figure 8- Channel and dyke routes	17

Figure 9- Roads and utilities	18

Figure 10- Land use	19

Figure 11- Intermediate catchments with no upstream dam	21

Figure 12- HEC-HMS model for block 06	23

Figure 13- (a) Q200 (m3/s)	26

Figure 14- (b) Q200 (m3/s)	27

Figure 15- inflow and outflow hydrographs for all dams	30

Figure 16-OQ pipeline and Highway constraints	32

Figure 17- Channel in fill condition	33

Figure 18- Channel in cut condition	33

Figure 19- Existing wadi vs Design channel depth	35

Figure 20- Channel width	36

Figure 21- Channels and Dykes Summary (Total 383 km)	37

Figure 22- Channel Width Distribution	39

Figure 23- Channels’ name	40

Figure 24- Channels’ piecewise label	41

Figure 25- 3D view of implemented channels	49

Figure 26- Outfalls in the channel system	50

Figure 27- cross drainage structures	54

Figure 28- Dykes Height	65

# Executive Summary

# Hydraulic Design of Channels– Block 6

The study area encompasses two governorates in Northern Oman: North and South Al Batinah. The Ministry of Agriculture, Fisheries Wealth and Water Resources (MAFWR) commissioned Renardet S.A. & Partners Consulting Engineers L.L.C. in joint venture with Artelia Muscat to undertake this consultancy.

The full project title is: “Consultancy Services for Study and Prepare Comprehensive Master Plan for Flood Protection Dams, River Training Schemes and Updating of Flood Maps and Stormwater Drainage System in North and South Al Batinah Governorates, Sultanate of Oman.” The project encompasses dams, dykes, and stormwater channels designed to safeguard urbanised areas, industrial zones, critical infrastructure, and future developments from flood damage.

This report documents the hydraulic design of flood protection and stormwater channels for Block 6. All channels are designed with the upstream dams fully in place, as these dams play a critical role in attenuating peak inflows. The target design is the 200-year return period flood. Attenuated and routed dam outflows are conveyed through the channel network to reach the sea, providing an integrated flood protection system. Dyke heights are determined from HEC-RAS 2D hydrodynamic simulations. The channel design process follows a systematic, multi-step methodology as summarised in table below and following chart.

Table 1- Channel Design Process – Block 6

| Stage | Title | Key Components / Methods |
| --- | --- | --- |
| 1 | Project Input | - Design Rainfall - Topography - Land Use - Dam Characteristics (height, reservoir, spillway/outlet) |
| 2 | Catchment Analysis | - HEC-HMS Subbasin Delineation - Area Reduction Factor (ARF) - SCS CN - Time of Concentration (TC) - Reservoir and Reach Routing parameters - Diversions |
| 3 | Rainfall–Runoff Simulation (HEC-HMS) | - 200-yr Peak Flow (and other return periods) - Inflow/Outflow Hydrographs |
| 4 | Channel Alignment | - HEC-RAS 2D results - Wadi Geometry to adjust the channel dimension - Site Visit - Client/Authority Input |
| 5 | Hydraulic Channel Design (SewerGEMS) | - Manning's Eq - Depth - Slope - Velocity - Freeboard - Culvert Sizing - Constraints (OQ pipeline, Bridge\Culvert) |
| 6 | Performance Assessment (HEC-RAS) | - Dyke Heights - Culvert\Bridge capacity assessment - Outfalls - System Verification - 200-yr Protection Check |
| 7 | Outputs | - Channel Dimensions - Dyke heights - Culvert\Bridge capacity - Solution for bottleneck (outfalls, narrow channels) |

Figure 1- Channel Design Process – Block 6

## Hydrology

A comprehensive, nationwide hydrology analysis is conducted by MAFWR recently. This study includes all the rainfall stations analysis, data cleaning, trend and outlier tests, gap filling, distribution fitting and eventually Regional Frequency Analysis (RFA) to calculate rainfall depth in different regions.

Therefore, from this study, the data used include the 24-hour design precipitation depth and patterns, for different return periods to perform rainfall-runoff simulation.

Table 2- Design Rainfalls

| Recurrence Interval | Depth (mm) |
| --- | --- |
| 2 | 27.6 |
| 5 | 55.3 |
| 10 | 105.5 |
| 25 | 137.5 |
| 50 | 168.1 |
| 100 | 205.4 |
| 200 | 251.0 |
| 500 | 327.2 |
| 1000 | 399.9 |
| 2000 | 488.7 |
| 5000 | 639.1 |
| 10000 | 685.1 |
| PMP | Grid (500-1840) |

The Probable Maximum Precipitation (PMP) is estimated using the storm transposition and moisture maximisation method as recommended by the World Meteorological Organisation (WMO). This approach involves identifying the most severe historical storm events in the region, transposing them to the catchment of interest, and maximising the precipitable water content to its theoretical upper limit based on atmospheric thermodynamic principles. The resulting PMP values represent the greatest depth of precipitation that is physically possible for a given duration and area, providing the basis for the design of critical hydraulic infrastructure. In this study, the nationwide PMP analysis conducted by MAFWR provides the 24-hour design rainfall depths used as input to the rainfall–runoff model. The dam spillways are sized based on probable maximum flood (PMF).

Figure 2-PMP grid

Figure 3- Rainfall Distribution Patterns

The Area Reduction Factor (ARF) accounts for the spatial variability of rainfall over a catchment. Point rainfall measurements tend to overestimate the average areal rainfall depth for large catchments, because intense storm cells rarely cover the entire catchment simultaneously. The ARF, a dimensionless factor ≤ 1, is applied to design point rainfall to obtain the spatially averaged areal rainfall. In this project, ARF values are derived from the nationwide hydrology study and applied to all catchments. This is especially significant for larger drainage areas in Block 6, where the ARF meaningfully reduces computed peak discharges.

Table 3- Area reduction factor (ARF)

| Area (km2) | ARF |
| --- | --- |
| 1 | 1 |
| 5 | 0.994 |
| 10 | 0.987 |
| 20 | 0.976 |
| 30 | 0.965 |
| 40 | 0.956 |
| 50 | 0.948 |
| 100 | 0.913 |
| 200 | 0.864 |
| 500 | 0.776 |
| 1000 | 0.694 |
| 2000 | 0.606 |

Figure 4 – Aea Reduction Factor Curve

The Curve Number (CN) grid was developed through the integration of two high-resolution global datasets: the ESA WorldCover 2020 (10 m) and the ISRIC SoilGrids (250 m). The ESA WorldCover product provides a refined classification of land use and land cover (LULC), allowing for a precise assessment of surface roughness and vegetation intercept. To account for subterranean influences on runoff, SoilGrids data was processed to determine Hydrologic Soil Groups (HSG) based on soil texture and drainage properties. The datasets were spatially aligned and combined via a standardized SCS (Soil Conservation Service) lookup table to generate a spatially distributed CN layer representative of the study area’s hydrologic response.

Figure 5- Curve Number grid